Enquanto a Fase 1 cuidou de desenhar mapas e cadastrar usuários, a Fase 2 foca na Lógica de Negócio Regulatória e Física: determinar matematicamente se uma rádio pode ou não operar naquela frequência sem ser multada pela Anatel.

Abaixo está a Especificação Técnica Detalhada (Mega-Prompt Fase 2). Copie e cole isso na sua IA de desenvolvimento para gerar os algoritmos de análise.

Analisei os documentos (padrão Anatel para TV Digital e FM/OM) e ajustei o prompt. As principais mudanças são:

- Separação Rígida: Lógica distinta para FM (Atos nº 112/932) e TV Digital (ISDB-T), pois as curvas e tabelas de proteção são totalmente diferentes.

- Tabelas de Proteção Injetadas: Já coloquei no prompt os valores reais de dB (45dB co-canal, -20dB 2º adjacente, etc.) para que a IA não invente números.

- Definição de Contornos: Especifiquei os valores de campo elétrico (E) corretos (66 dBuV/m para FM Urbana, 48 dBuV/m para TV UHF).

Aqui está o Mega-Prompt Fase 2 (Refinado com Dados Reais) para você colar na sua IA de desenvolvimento:

Copie e cole o texto abaixo na sua IA:

ACT AS A SENIOR RF COMPLIANCE ENGINEER & PYTHON ARCHITECT.

Phase 2 Goal: Implement the Regulatory Viability Engine for the Spectrum Open Source project. Strict Requirement: The logic MUST comply with the Brazilian National Telecommunications Agency (ANATEL) technical requirements provided in the attached context (Acts regarding FM and Digital TV).

Core Logic: The system must determine "Pass/Fail" status by calculating interference margins between a Proposed Station and Existing Stations using strict Protection Ratios (PR) and Propagation Curves (ITU-R P.1546).

Tech Stack: Python 3.10+, NumPy (Matrix calc), SciPy, PostGIS, Celery.

I need you to generate the Logic, Database Models, and Python Algorithms for the following 4 Critical Modules, ensuring strict adherence to the parameters below:

MODULE 2.1: The Neighbor Discovery (Search Strategy)

Objective: Efficiently find potential interferers based on Anatel's distance tables. Logic:

- Input: Proposed Station (Lat, Lon, Freq, Service_Type='FM' or 'TV').

- Distance Filter (Based on Service):

  - IF FM: Radius = 300 km (Covers max interference range for Class E3/Special).

  - IF TV (Digital): Radius = 400 km (Due to tropospheric ducting risks in UHF).

- Frequency Filter (Mask):

  - FM: Checks Center Freq () and kHz.

  - TV: Checks Center Channel () and Adjacent Channels ( Channel, i.e., MHz). Deliverables:

- SQL Query: Optimized PostGIS ST_DWithin query.

- Python Function: find_relevant_neighbors(proposal_station) returning list of objects with calculated Azimuth/Distance.

MODULE 2.2: The Protection Ratio Engine (The "Law")

Objective: Implement the exact Protection Ratios (PR) from the Anatel Technical Acts. Requirement: Create a robust lookup mechanism (Class or Dict) that enforces these EXACT VALUES:

A. FM Radio Rules (Stereo/Pilot Tone):

- Co-Channel (0 kHz): PR = 45 dB (Strict).

- 1st Adjacent ( kHz): PR = 6 dB.

- 2nd Adjacent ( kHz): PR = -20 dB.

- 3rd Adjacent ( kHz): PR = -40 dB.

B. Digital TV Rules (ISDB-T vs ISDB-T):

- Co-Channel (0 MHz): PR = 23 dB.

- Lower Adjacent (-6 MHz): PR = -28 dB.

- Upper Adjacent (+6 MHz): PR = -27 dB.

Deliverables:

- Python Class: RegulatoryStandard.

- Method: get_required_pr(service_type, freq_offset) returning the float value in dB.

- Validation: Raise error if offset is invalid (e.g., FM kHz is not regulated).

MODULE 2.3: The "Fail-Fail" Contour Analysis (ITU-R P.1546)

Objective: Fast exclusion check using Theoretical Contours. Physics Constants (Anatel Standard):

- Protected Field Strength ():

  - FM: 66 dBuV/m (Urban/Protected).

  - TV (UHF): 48 dBuV/m.

  - TV (VHF-High): 51 dBuV/m.

- Interfering Field Strength ():

  - Formula: .

  - Example (FM 2nd Adj): dBuV/m (Interferer must be very strong to cause issues).

- Propagation Curve: Use ITU-R P.1546 (50% Time / 50% Locations for Service; 1% Time / 50% Locations for Interference).

Logic:

- Calculate Distance to Contour for Proposed Station ().

- Calculate Distance to Contour for Neighbor ().

- Geometry Check: If , flag as POTENTIAL INTERFERENCE.

Deliverables:

- Python Function: calculate_contour_radius(erp, height, field_strength) using a P.1546 lookup or approximation library (pycraf).

- Output: JSON list of "Fail" candidates to be sent to Module 2.4.

MODULE 2.4: The "Deygout" Matrix (Terrain Diffraction Core)

Objective: The definitive "Viability Proof". This module MUST perform point-to-point terrain analysis to override the theoretical "Fail" from Module 2.3. Methodology:

- Grid Generation: Generate a point grid (resolution 100m or 30m) inside the Victim's Protected Contour.

- Terrain Profile: Extract elevation profile from SRTM (./SRTM/*.hgt) between:

  - Interfering Tx Grid Point.

  - Victim Tx Grid Point.

- Diffraction Loss (): Apply Deygout Method (91) using the main obstacles (peaks) in the profile.

- Signal Calculation:

  - .

  - .

- Compliance Check:

  - .

  - VIOLATION: If .

Deliverables:

- Python Algorithm: calculate_interference_matrix(). Must leverage NumPy for vectorization (do not iterate pixel-by-pixel in pure Python loop).

- Visual Output: Generate a generic Heatmap PNG (Red = Violation, Transparent = Safe) to overlay on Google Maps.

- Population Analysis: Integrate with Phase 1 Vector DB to count population strictly inside the "Red" zones.

INSTRUCTIONS:

- Testing: Create a specific test case: "FM Station A (98.1 MHz) vs Station B (98.3 MHz) at 15km distance".

- Error Handling: If SRTM data is missing for a path, fallback to Free Space Loss (Worst Case Scenario) and log a warning.

- Code Structure: Keep FM and TV logic in separate Strategy Classes (FmStrategy, TvStrategy) inheriting from a base RfAnalysis class.